Islamic University of Gaza

Computer Engineering

بسم الله الرحمن الرحيم

Embedded Systems

Homework #2

Student Name      Akram Hatem Abo Jbara

Student No.:         120200602

Instructor Name:   Eng. Amal Abu Jasser

Day:                      Saturday

Date:                     29/Nov/2024

Use a Common- Anode 7-Segment Display for 4 digits to count from 555 to 0, when reaching to 0 a red led connected to port 1 must shine.

Code(Text and Screenshot):

#include <LPC11xx.h>

#define GPIO0DIR (*((volatile unsigned long *)0x50008000))

#define GPIO0DATA (*((volatile unsigned long *)0x50003ffc))

#define GPIO1DIR (*((volatile unsigned long *)0x50018000))

#define GPIO1DATA (*((volatile unsigned long *)0x50013ffc))

#define GPIO2DIR (*((volatile unsigned long *)0x50028000))

#define GPIO2DATA (*((volatile unsigned long *)0x50023ffc))

int main (void){

int seven_seg_encoder [] = {

0x40,

0x79,

0x24,

0x30,

0x19,

0x12,

0x02,

0x78,

0x00,

0x10

};

int num=555;

int i =0;

GPIO2DIR |= 0b1111111;

GPIO0DIR |= 0b111100;

GPIO1DIR |= 0b10000;

GPIO1DATA = 0b00000;

while (1) {

GPIO0DATA=0b000100;

GPIO2DATA = 0x40;

for (i =0; i<20000; i++);

GPIO0DATA=0b001000;

GPIO2DATA = seven_seg_encoder[(num/100) %10];

for (i =0; i<20000; i++);

GPIO0DATA=0b010000;

GPIO2DATA = seven_seg_encoder[(num/10) %10];

for (i =0; i<20000; i++);

GPIO0DATA=0b100000;

GPIO2DATA = seven_seg_encoder[(num) %10];

for (i =0; i<20000; i++);

num--;

if(num==0){

GPIO1DATA = 0b10000;

GPIO2DATA = 0b1000000;

break;

}

}

return 0;

}

Result

Thank You